EGB345 Control and Dynamic Systems	     	JJF/25

Servo Motor Control Design Report

| Author Details | Darcy Stewart N11977965 |
| --- | --- |

Answers

| Design Summary | Summary of parameters and task outcomes are: Task 1; Km; 6.8528. Alpha; 3.2737. KTask1; 0.8240. Task 3; Kfo; 0.6869. Kso; 0.4966. Task 4; Pc; -3.2737. Zc; -6.54. Klead; 3.2772. |
| --- | --- |
| Task 1 | The behaviour of the open loop transfer function was initially analysed through taking the inverse Laplace transform of a step input. The inverse Laplace transform yielded the function of: r(t)⊗g(t)=u(t)(2.093t-0.63913+0.6394e-3.2737t) This step response indicates that in open loop the motor angle will increase indefinitely, suggesting that providing a step input would simply turn the motor on and make it turn in the “positive” direction. This analysis was supported by a plot of the step input in MATLAB included below. The graphed step response shows that the motors angle would continue to increase as time increased, supporting the mathematical analysis. Following this the model of a closed loop system with negative feedback was developed in the interest of control. The model was developed using the formula of Ges=11+GsHs where Ge(s) is the transfer function of the feedback system, G(s) is the transfer function of the motor and H(s) is the gain of the feedback. For the purposes of this report H(s) is assumed to be 1. This model is correct however for an adjustable model a gain characteristic must be incorporated to tune the response to the desired output. This gain component cascades with the motor transfer function giving a new transfer function of Gs=KG(s) where Gms represents the motor transfer function. When this was expanded and rearranged the closed loop transfer function was found to be: Ges=K×Kms2+αs+K×Km Following this analysis was performed to see what the resultant system without any gain would be. This was achieved through setting K to be 1, with all of the values substituted this resulted in a transfer function of Ges=6.8528s2+3.2737s+6.8528. The transfer function is in the form of a second order transfer function. The first stage of analysis was to evaluate the poles to determine the type of system. The poles of the system were found to be s= -1.637±2.033j, complex poles indicate that the system is underdamped. Using the general second order system values the step response behaviours can be approximated, these values were calculated below. ωn=6.8528=2.618 ζ=3.27372ωn=0.625 Ts=40.625×2.618=2.94 seconds %OS=100eζπ1-ζ2=100e-2.517=8.069% Numerical analysis determines that the system with overshoot by 8.069% and reach stability at 2.94 seconds. To confirm this a MATLAB simulation was plotted below: The simulation confirms the properties derived numerically. Following this it was decided to modify the gain to allow for a 5% overshoot. The key value referring to overshoot is the damping ratio ζ. This was calculated using the formula ln%OS100=-ζπ1-ζ2. This found that the value of ζ required is 0.6901. Following this ωn must be calculated to allow for the adjusted ζ value. ωn=3.27372×ζ=2.372 The value of K was calculated that would achieve this value of ωn. K=ωn26.82528=0.824 The resultant systems step response was plotted below: The step response of the 5% overshoot model behaved as predicted with a max overshoot of 5%. MATLAB CODE: %%1.1 clear all; Km = 6.8528; Alpha = 3.2737; num = [Km]; den = [1 Alpha 0]; open_loop = tf(num,den); t = linspace(0,3.5,10000); open_loop = step(open_loop,t); figure; plot(t, open_loop) title('Open Loop Step Response'); xlabel('Time (s)'); ylabel('Amplitude'); %% 1.5 closed_loop = tf([Km],[1 Alpha Km]); figure step(closed_loop,t) title('Closed Loop Step Response (K = 1)') xlabel('Time (s)') ylabel('Amplitude') %% 1.6 KTask1 = 0.824; five_pecernt_overshoot = tf([KTask1*Km],[1 Alpha Km*KTask1]); figure step(five_pecernt_overshoot,t) title('Closed Loop Step Response (5%Overshoot)') xlabel('Time (s)') ylabel('Amplitude') |
| Task 2 | K value. 1.8; s=1.637±-3.107j. 1.6; s= -1.637±2.878j. 1.4; s= -1.637±2.630j. 1.2; s= -1.637±2.355j. 1; s= -1.637±2.043j. 0.8; s= -1.637±1.674j. 0.6; s= -1.637±1.197j. 0.4; s= -1.637±0.248681. 0.2; s= -2.781, s= -0.493. 0.1; s= -3.04894, s=-0.22476. Hand drawn poles at gain values MATLAB Code: K_16 = tf([1.6*KTask1*Km], [1 Alpha 1.6*KTask1*Km]); K_10= tf([KTask1*Km], [1 Alpha KTask1*Km]); K_02 = tf([0.2*KTask1*Km], [1 Alpha 0.2*KTask1*Km]); figure hold on step(K_16,t) step(K_10,t) step(K_02,t) hold off legend ('1.6','1','0.2'); title('Step Resopons (Varying K)') |
| Task 3 | The addition of the added poles adds a layer of complexity as there is not easy way to calculate settling time and percent overshoot. As a result, root locus was determined to be the most efficient method for determining the gain required to get the desired response. The root locus of the fast pole system was plotted after calculating some key values. These values include the asymptote point (σ0), asymptote angels (θ0), zeta ζ, breakaway point (σ) and the point where the root locus crosses the imaginary axis (ω), this is completed below. σ0=∑finite poles-∑finite zeros#finite poles-#finite zeros=0-3.2737-32.7373-0= -36.013=-12 θ0=2k+1180#finite poles-#finite zeros=60°,180°,300° ζ=-ln%OS100π2+ln2%OS100= -ln5100π2+ln25100=0.6901 Breakaway Point Calculation 0=1σ+3.2737+1σ+1σ+32.737 0=σσ+32.737+σ+3.2737σ+32.737+σσ+3.2737σσ+3.2737σ+32.737 σ= -22.4313, -1.5938 Imaginary Axis Cross point -1=Km×10α×Kjωjω+αjω+10α jωjω+αjω+10α= -Km×10α×K jω10α2+ω2-ω2α-10ω2α= -Km×10α×K jω10α2-ω2Km×10α-ω2α-10ω2αKm×10α= -Km Since we are looking for the point at which the root locus crosses the imaginary axis that is all that we care about, therefore. 10α2-ω2Km×10α=0 10α2=ω2 ω=10×3.27372 ω=±10.35 Therefore, the root locus crosses the imaginary axis at ±10.35j. To truly utilise the root locus to determine a suitable gain for a percentage overshoot requirement the angle that the damping ratio will follow on the root locus must be found as the desired poles will be found at the intersection between the root locus and damping ratio line. This is calculated through taking the inverse cosine of ζ. θ=cos-1ζ θ=cos-1-0.6901 θ=46.4° As the root locus is being drawn by hand and approximation of 45° will be sufficient. With these values calculated the root locus was sketched. The intersection between the root locus and the damping ration line was found to be at -1.5±1.5j. This indicates that the desired poles exist at s= -1.5±1.5j, using this gain can be calculated using the formula: K=lims→p1Gs=lims→-1.5+1.5jss+αs+10α10×6.88528×3.2737=0.686919 The step response of the system with this gain was then plotted below: It can be seen that the max percentage overshoot sits at 4%. This is close to the target and I considered accurate given that several approximations were required to draw the root locus by hand. This process was completed following the same process for the extra slow pole to calculate the gain required for a 5% overshoot: σ0=∑finite poles-∑finite zeros#finite poles-#finite zeros=0-3.2737-6.54743-0= -9.82113=-3.2737 θ0=2k+1180#finite poles-#finite zeros=60°,180°,300° ζ=-ln%OS100π2+ln2%OS100= -ln5100π2+ln25100=0.6901 Breakaway Point Calculation 0=1σ+3.2737+1σ+1σ+32.737 0=σσ+32.737+σ+3.2737σ+6.5474+σσ+3.2737σσ+3.2737σ+6.5474 σ= -5.164, -1.384 Imaginary Axis Cross point -1=Km×2α×Kjωjω+αjω+2α jωjω+αjω+2α= -Km×10α×K jω2α2+ω2-ω2α-2ω2α= -Km×2α×K jω2α2-ω2Km×2α-ω2α-2ω2αKm×2α= -Km 2α2-ω2Km×2α=0 2α2=ω2 ω=2×3.27372 ω=±4.63 θ=cos-1ζ θ=cos-1-0.6901 θ=46.4° Once again using these values the root locus was sketched: The desired pole was found to be at s=1.2±1.2j. K=lims→p1Gs=lims→-1.2+1.2jss+αs+2α2×6.88528×3.2737=0.496633 The step response was plotted to evaluate the feasibility of the model: The system peaks at a 4% overshoot demonstrating the feasibility of the model given that approximations had to be made to allow for hand sketching. To evaluate how the gains of each of the systems acted on each other new step responses were plot to allow for inspection. After evaluating the properties of each it was decided that the gain of Kfo has to most stability across the three systems, as it still has a fast-settling time while having an overshoot of seven percent or less. As such it is considered the most robust system. MATLAB Code: %% Kfo = 0.686919; Gfo = tf([10*Alpha*Km*Kfo],[1 Alpha+10*Alpha 10*Alpha^2 Kfo*10*Alpha*Km]); figure step(Gfo) title('Step Response Extra Fast Pole With K = 0.686919') xtitle('Time (s)') ytitle('Amplitude') %% Kso = 0.496633; Gso = tf([2*Alpha*Km*Kso],[1 Alpha+2*Alpha 2*Alpha^2 Kso*2*Alpha*Km]); figure step(Gso) title('Step Response Extra Slow Pole With K = 0.496633') xtitle('Time (s)') ytitle('Amplitude') %% task 3.5 GO_fast = tf([Kfo*Km],[1 Alpha Km*Kfo]); GO_Slow = tf([Kso*Km],[1 Alpha Km*Kso]); Gfo_original = tf([10*Alpha*Km*KTask1],[1 Alpha+10*Alpha 10*Alpha^2 KTask1*10*Alpha*Km]); Gfo_slow = tf([10*Alpha*Km*Kso],[1 Alpha+10*Alpha 10*Alpha^2 Kso*10*Alpha*Km]); Gso_fast = tf([2*Alpha*Km*Kfo],[1 Alpha+2*Alpha 2*Alpha^2 Kfo*2*Alpha*Km]); Gso_Original = tf([2*Alpha*Km*KTask1],[1 Alpha+2*Alpha 2*Alpha^2 KTask1*2*Alpha*Km]); figure step(GO_fast); title('Original System Step Response With Kfo Gain') xlabel("Time") ylabel('Amplitude') figure step(Gso_fast); title('Slow System Step Response With Kfo Gain') xlabel('Time (s)') ylabel('Amplitude') figure step(Gfo_slow); title('Fast System Step Response With Kso Gain') xlabel('Time (s)') ylabel('Amplitude') figure step(GO_Slow); title('Original System Step Response With Kso Gain') xlabel('Time (s)') ylabel('Amplitude') figure step(Gso_Original); title('Slow System Step Response With Original Gain') xlabel('Time (s)') ylabel('Amplitude') figure step(Gfo_original); title('Fast System Step Response With Original Gain') xlabel('Time (s)') ylabel('Amplitude') |
| Task 4 | The closed loop poles of the transfer function Gs=Kmss+α can be found using through treating the denominator as a quadratic function and solving for the roots. ss+α=0 ∴s=0 or s= -α= -3.2737 As the system only has two poles the root locus travel to the centre before increasing to ∞ and -∞ at angles normal to the real axis. As such the desired poles can be found through calculating the breakaway point and applying the formula pd=σ±σtanθj where θ is the angle calculated by the damping ratio ζ, which for the case of G(s) is 46.4°. σ=∑finite poles-∑finite zeros#finites poles-#finite zeros=0-3.2732-02-0=-1.637 pd= -1.637±-1.637tan46.4j= -1.637±1.719j Since it is desired for the settling time to be halved it is crucial to know what the original settling time. Ts=4σ=41.637=2.4439 With this calculated the value of σ required to produce a Ts of 1.2219 can now be calculated. σ=4Ts=41.2219=3.2737 With this in mind the lead compensator must adjust the root locus such that its breakaway point is on 3.2737 on the real axis. The dynamic compensator is the part of the system responsible for this shift. In this case a lead compensator is best utilized as it is a straightforward implementation for a second order system. The lead compensator takes the form of s+zcs+pc where zc and pc represent the zero and pole of the compensator. To achieve this shift of the breakaway point the dynamic compensator has a zero set at same point the outer pole effectively neutralising the majority of its effect on the system. As such zc=3.2737. As the most positive pole sits directly on the origin pc must sit such that the pc2=3.2737. As a result pc=6.5474. With this in mind the dynamic compensator is added to the system producing a transfer function of: Gs=s+3.2737s+6.54Kmss+3.2737=Kmss+6.54 At this point the poles for the desired overshoot can be calculated through pd=-3.2737±3.2737tan46.4j= -3.2737±3.43j With this the suitable gain for this system can be calculated through: K=¦lims→-3.2737+3.43jss+6.54Km¦=3.27717 The step response of this was plot to evaluate whether the compensator is achieving the desired response: It can be seen that the model with the lead compensator adjusts the response to take half the time to settle and still has an overshoot of 5%, thus demonstrating that the lead compensator is successful. MATLAB Code: %% task 4.2 Klead = 3.27717; Glead = feedback(tf([Klead*Km],[1 6.54 0]),1); figure step(Glead,t) |